4-CIVIL AND STRUCTURAL SCOPE

4.1	GENERAL DESCRIPTION

4.2	JOB DESCRIPTION

4.3	CIVIL/STRUCTURAL EQUIPMENT

4.4	TESTING AND FIELD INSPECTION

CIVIL AND STRUCTURAL SCOPE

  - GENERAL DESCRIPTION

4.1.1	Scope

This Scope of Work covers the minimum requirements for all civil works of ARAB Gas Pipeline project phase II. CONTRACTOR Scope shall include engineering, personnel, supervision, materials, consumable and construction equipment to construct, and deliver complete all the civil and structural works for the project to the COMPANY in accordance with RFQ Requirement and Specification.

The civil and structural works are located alongside the pipeline, laid between:

From AL FOREQLS area to Aleppo PRMS passing through  BVS's as well as in the Block Valve Stations, Tie-ins and F.M Stations.

CONTRACTOR shall be responsible for all clean-up, finish grading and restoration of disturbed areas.

4.1.2	Codes, Standards and Documents

The civil and structural activities, including foundation design and equipment installation shall conform to all applicable RFQ specifications and schedules, Syrian Arab Code- United States and International Codes and Standards including but not limited to that specified in the “EXHIBIT-G, Data References and CONTRACT’s General Specifications”.

In case a discrepancy arises or exists between codes, standards, drawings and documents, the most severe shall govern.

4.1.3	Drawings and Specification Lists

CONTRACTOR shall provide all the civil and structural drawings, specification lists and Documentation and is responsible to show the all details of the work and to ensure the drawings are correct and complete before beginning the work. All drawings provided in this RFQ tender book are for information only.

4.1.4	Materials and Equipment

CONTRACTOR shall provide all materials, personnel, tools and construction equipment to construct all the civil and structural works in accordance with the RFQ Requirement, and Specifications.

CONTRACTOR shall identify critical items of equipment and other materials and shall make appropriate arrangements to order and have such items delivered to meet the required overall completion date. This includes forms, concrete, rebar, embedded steel, anchor bolts, structural steel, bolting, etc.

4.1.5	Construction

CONTRACTOR shall provide all personnel, supervision, co-ordination, tools and materials as well as competent knowledge of construction for the forming, installation, testing and inspection of all the works.

CONTRACTOR shall execute all works in a workmanlike manner so as to present a neat and finished appearance when completed.

CONTRACTOR shall become familiar with the Construction Drawings of all trades and when laying out the work with regard to structural and mechanical features.

CONTRACTOR shall provide for approval a construction schedule which will take into full consideration the availability or delivery of all equipment and materials, the target completion date required by the COMPANY.

CONTRACTOR shall arrange his own inspection team and equipment for the quality control of the whole work. It shall be noted that although the COMPANY will appoint its own independent inspection agent, the responsibility for meeting the appropriate quality standards will remain with the CONTRACTOR.

4.2	JOB DESCRIPTION

4.2.1	General

This job description covers the basic requirements for the Civil and Structural works and for the supply and installation of concrete and structural equipment materials.  It is intended to supplement the construction drawings and to serve, as an installation guide for activities not detailed.

All work shall be done in accordance with RFQ requirements, Codes, Standards and Specification.

CONTRACTOR shall provide all materials, personnel, tools, construction equipment and all other requirement for the execution of Civil and Structural works of the Project.

CONTRACTOR shall provide all civil and structural works suitable to resist the aggressive see environment

4.2.2	Detailed Scope of Work

CONTRACTOR shall be responsible for all Earthworks, Concrete, Structural Steel and Masonry required for the construction of ARAB Gas Pipeline project phase II CONTRACTOR shall provide all construction equipment needed to complete the WORK in accordance with the schedule, specifications and requirement. This work shall be performed at the following locations, but not be limited to:

      - Fiscal Metering Station (FMS)

CONTRACTOR’s scope of work at the Fiscal Metering Station includes, but not be limited to the following:

-	Stripping of all surface soil to a depth to remove all loose dirt, roots, vegetation, and/or other extraneous materials from the confine roadway, walkway, ditches, parking area and other surface area.

- Site preparation and grading, site survey and soil report and drawings.

- Selected fill materials shall be placed instead of top soil and compacted to 98% of standard proctor max dry density

- Site fencing using masonry- reinforced concrete-steel materials (the area to be prepared and fenced shall be of 20000 m2 as minimum for Al Foreqls FMS.

- Build new building with local heating and air conditioning system as described further in this scope and showing on the attached drawings No.     (TYPICAL)

- The reinforced concrete roof of building shall be extended as minimum 100 cm at all sides of building and all roof slab parapet must be reinforced concrete.

- The outside surface of concrete roof of buildings should be covered with water- heat insolating layers.

External concrete block walls surface for buildings shall be covered with white marble tiles of 30 mm minimum thickness.

-	Concrete sidewalk around all building shall be provided using colored inter-lock tiles.

- The following building types shall be constructed of reinforced concrete blocks in the FMS’s area.

- Control building which shall have one floor 400 m2 area as minimum for Al Foreqls FMS.

- , see Dwg. (TAYPICAL)

II- Work Shop building which shall have one floor 200 m2 areas as minimum FOR Al Foreqls FMS.

III-Guard house beside the main gate of single 20 m2 room with Split type                air heating / conditioning unit

-	Construction of concrete foundation for the FMS equipment and skids/packages.

-            Car parking area with sunshade for ten (10) cars as minimum for al Foreqls FMS

-              Construction of sunshade for diesel generator.

-	Construction of pipe sleeper(s)/pipe support(s)

-	Supply potable water from the water system in living area at Al Foreqls (about 500 m from the fence of the FMS) to the FM Station by using required galvanized pipes .

-	Supply and installation of (30 m3 for al Foreqls FMS of Potable water tanks on the steel base to supply sufficient pressure to the system. Distribute the water line to the all skids units, control and work shop buildings and guard house.

- The control and work shop building shall have drinking refrigerator, connected to the potable water system.

-	One septic tank (for 25 persons) as per specification shall be provided as minimum.

-           The sewage drain system of the station shall be connected to the nearest existing sewage in the area (or leach field system if no sewage there).

-	Evaporation bond of not less than 20 m3 evaporation areas to which all skids drains shall be routed.

-             The skid drains shall be routed to the to the drain route

-	The final leveling of the location shall be as min. (30 cm) higher than surrounding ground level with drain slope.

-	The final layer of the area should be as min (20 cm) gravel and sand compacted to98% of standard proctor max dry density, also side walk of concrete tiles (100x50x7 cm) dimension from control building to all skids shall be provided.

-             grouting where is required

- painting

- Testing and documentation.

-	Site clean up and restoration.

4.2.2.2   Block Valve Stations (BVS) and pigging station areas

CONTRACTOR shall perform and execute the following activities for all BVS's and Pigging station areas

- Site preparation and grading.

- Stripping of all surface soil to a depth to remove all loose dirt, roots, vegetation, and/or other extraneous materials from the confine roadway, walkway, ditches, parking area and other surface area.

- Selected fill materials shall be placed instead of topsoil and compacted to 98% of standard proctor max dry density.

- Site fencing, 2 meters high using reinforced concrete for the foundation and columns and honeycomb concrete block for the walls placed between columns, covered with trowel led cement plaster and sprayed with colored white cement / sand mixture. Barbed wire- two inch ( 12-1/2gage) galvanized wire with 0.080 inches ( 14 gage )four point round barbs spaced at 5 inches on centers.

Barbed wire extension arms suitable for three strands of barbed wire spaced 6 inches apart and set at a 45 degree angel, arms extend 1 foot vertically above top rial. Arms shall withstand a 300 lb load applied at the outer strand of barbed wire.  (the area to be prepared and fenced shall be of 1000 m2 for each BVS and 3500 m2 for pigging station area as minimum)

- Construct BVS’s building and telecommunication building as showing on attached drawings no. (TYPICAL).

Guard house beside the main gate of single 20 m2 room with split type

Air heating / conditioning unit

- Concrete sidewalk around the telecommunication building shall be provided. Telecommunication room and batteries room) TYPICAL ) shall have split type air conditioning at all BVS and pigging area, which shall be powered from the Syrian overhead power line.

- External concrete block walls surface for telecommunication and battery buildings shall be covered with white marble tiles of 30 mm minimum thickness.

- The outside surface of concrete roof of telecommunication and battery buildings should be covered with water- heat insolating layers.

- Construction of concrete foundation for skids and packages. Construction of pipe sleepers/pipe supports

- The final leveling of the location shall be as min. (30 cm) higher than surrounding ground level with drain slope.

- The final layer of the area should be as min (20 cm) gravel and sand, compacted  to98% of standard proctor max dry density

- The floor of telecommunication building shall be as minimum (30 cm) higher than final layer of BVS location.

-             Grouting where is required.

- Testing and documentation.

- Painting.

- Site clean up and restoration.

      - Anchor Blocks and main pipeline crossings

CONTRACTOR shall execute the following activities at anchor blocks and pipeline crossing area, includes but not limited to

- Site preparation and grading

- Temporary gate and crossing facing

- Construction of concrete anchor block.

- Excavation/boring for under road, river or railway crossing and pipeline casing.

- Concrete reinforcing pad(s) under all major road crossings (for open ditch).

- pipeline markers and painting

- Grouting where is required.

- Testing and documentation

- Site clean up and restoration.

      - Roads

CONTRACTOR shall construct and build (30000 m2) asphalt roads at several locations of project, also shall construct and build (15000 m2) of graveled roads as per specification and drawings attached, the cost of this activities shall be paid as actual volume performed based on the unit rate price provided, however the total cost shall be part of CONTRACTOR’s lump sum price. The roads shall be executed as per specifications and attached drawing no: (TYPICAL)

      - Finishes

- Exterior doors and door frame shall be made of steel, with good isolation and shall be provided with intruder switch and automatic door closure and door opening signal to be transmitted to SCADA system

- Windows and doors of buildings at FMS's shall be furnished with wire mesh

- As minimum, the following furniture (of first class and quality) shall be provided in the FMS's control buildings:

-	One wooden office table, three chairs and filing cabinet and computer for supervisor office.

-	One wooden office table, two chairs and filing cabinet for control room.

One wooden meeting table approximately 280x100 cm size and ten chairs and filling cabinet in the second office of control building.

4.3	CIVIL/STRUCTURAL EQUIPMENT

CONTRACTOR is responsible for providing all equipment required for the transport of material to site.  CONTRACTOR shall supply all on site equipment, materials and products required to complete the project as per specifications and RFQ requirements.

4.4	TESTING AND FIELD INSPECTION

CONTRACTOR is responsible for all testing and field inspection.

All materials and construction shall be tested and inspected for conformance with Codes, Standards and Documents provided in a/m par. 4.1.2

Inspection shall not relieve CONTRACTOR from any duties, obligations, responsibilities or liabilities under this CONTRACT.

  - CIVIL DRAWINGS

CONTRACTOR shall find in the “Exhibit-H, Drawings”. The civil drawings are provided as preliminary for information only.