8-SURVEY AND SOIL REPORT SCOPE

8.1	GENERAL DESCRIPTION

8.1.1	Scope

8.1.2	Codes, Standards and Documents

8.1.3	Drawings

    - Soil Sample Report

8.2	JOB DESCRIPTION

8.3	DETAILED SCOPE OF WORK.

8.3.1	Surveying Scope

8.3.2	Soil Testing Scope

8.3.3	Methodology and Schedule

8.3.4	Main P/L Survey-ALFOREQLS to Aleppo PRMS.

8.3.5	Survey and Right of Way

8-SURVEY AND SOIL REPORT SCOPE

  - GENERAL DESCRIPTION

    - Scope

This Scope of Work covers the minimum requirements for the ARAB Gas Pipeline project PHASE II. The CONTRACTOR's scope of work includes the supply of personnel, supervision, materials, consumables and equipment to provide the Pipeline Route Survey and the Soil Sample Report for the project in accordance with RFQ Specifications and Standards and Industry Codes and Standards.

The CONTRACTOR shall supply Pipeline Route Survey and Soil Sample Report and shall prepare AUTOCAD detailed pipeline alignment drawings, and detailed Typical Construction Pipeline Crossing drawings etc. However the COMPANY shall supply to the CONTRACTOR the typical crossing drawings and preliminary pipeline rout selection.. The CONTRACTOR is responsible to examine the COMPANY supplied drawings and data and report to the COMPANY any discrepancies and interference.

The Pipeline Route Survey and Soil Sample Report shall include the pipeline route the Block Valve Stations, Shutdown Valve, Scraper Receiver and Launcher, Fiscal Metering Stations and Anchor Block Sites etc.

    - Codes, Standards and Documents

The work shall be carried out in accordance with the applicable RFQ Standard Specifications and United States and International Codes and Standards but not limited to that specified in the “Exhibit-G, Data References and CONTRACT’S General Specifications”.

In case a discrepancy arises or exists between codes, standards, drawings and documents, the most severe shall govern.

    - Drawings

The CONTRACTOR’s Detailed Pipeline Alignment drawings shall include but are not limited to pipeline crossings, major/minor road and railway crossings, Wadi/Canal Crossings, River Crossings, power line, cables Crossings etc.

The Drawings and Documentation shall be provided as per AFPC Schedule-B, Engineering Documentation.

The following maps and drawings, as minimum should be prepared and provided by CONTRACTOR:

- Route maps Scale 1:10,000 length and 1/1000 width.

- Pipeline route to be Plotted on 1/50,000 , these maps Using Lambert coordinates.

- Key map for route maps Scale 1:500,000.

- Alignment Sheets Scale 1:1,000 Horizontal and 1:200 Vertical for a complete route.

# Property Plot drawings; scale plot to be approved by COMPANY

# Crossing drawings:

Separate drawings for each crossing with (Road, Railway, Wadi, Channel, cable etc) on a suitable Scale (1:100). See PL-CP, par. 16.3.2 for WADI categorization; however COMPANY should approve this categorization at site prior to execute the activity.

- CONTRACTOR shall prepare layouts for route map including all crossings in order to be approved by COMPANY.

The CONTRACTOR is responsible to collect and document the following information:

- Topographical Survey of a corridor of 100 m width (50 m each side of the Pipeline Centerline) and 200m width in critical area determine by the COMPANY.

- Slack Chainage along proposed Pipeline ditch line and Chainage notes for physical features, boundaries, road crossings, railroad crossings, Utility crossings, river and Wadi/Canal Crossings and buoyancy control Crossings.

- Longitudinal profile showing elevations for complete pipeline route. Each 50-m elevation should indicate the highest point and lowest point.

- Individual profiles and details required to design and prepare crossing drawings.

- Site information for all proposed pipeline facilities.

- Soil sample report includes soil characteristics, details for pipeline route. The CONTRACTOR shall drill at least one Boring (sondage) each one km in the depth of (10 m) to prepare soil test. In case the soil characteristic between two borings are different, then CONTRACTOR shall drill a third one between, to define the soil characteristic. In case the pipeline cross the slop area then a (20 m) depth drilled Boring should be executed, however, the CONTRACTOR shall avoid passing slops parallel to contour lines. In case it is possible to do that, the CONTRACTOR has to take necessary arrangements to ensure the stability of soil in the future and avoid the movement of the pipeline.

The COMPANY shall supply to the CONTRACTOR the preliminary pipeline rout selection and typical crossing drawings for information only.

    - Soil Sample Report

The Soil Sample Report shall be provided by the Contractor and is intended to describe the different soil types and layer depths that will be encountered along the Pipeline Route and Facility(s). However this report shall show all characteristics and details necessary to complete the detailed design of pipeline and alignment sheets, civil, Cathodic protection and mechanical drawings.

  - JOB DESCRIPTION

General

This job description covers the basic requirements for the Pipeline Route Survey and Soil Sample Report for the Pipeline Route and Facility(s). The survey is defined as the Engineering and Legal Survey which shall be performed by the CONTRACTOR.

All work shall be done in accordance with RFQ Standards and Specifications (Schedule PL-CP) - and United States and International Codes and Standards.

- The CONTRACTOR shall provide all materials, personnel, tools and construction equipment for the Pipeline Route Survey and Soil Sample Report for the Pipeline Route and Facilities.

- The total estimated route length for PHASE II is approximately 186 km.

- The COMPANY emphasizes that it takes no responsibility for the accuracy of the data presented on the preliminary pipeline route selection. The CONTRACTOR’s responsibility is to check the data, which are provided on the preliminary pipeline rout selection.

- The preliminary pipeline route starts as indicated in the attached drawings as the following:

- From AL FOREQLS STATION to ALEPPO PRMS.

- The CONTRACTOR shall be responsible for execution of the Pipeline Route Survey (Engineering and Legal Survey), Pipeline rout selection and Soil Sample Report, in cooperation with COMPANY’s representative(s).

- CONTRACTOR is fully responsible to define all the above, under ground crossing and to get all necessary permits from the deference authorities for executing all kind of works related to the crossing, topographical survey, soil testing etc. The COMPANY will provide the necessary support letters to the authorities.

- The CONTRACTOR shall be responsible to get all work permits from proper authorities of third parties alongside the pipeline rout etc.

  - DETAILED SCOPE OF WORK

    - Surveying Scope

Specific survey requirements are as follows:

| - River Crossings - Other crossings as specified | -Pipeline From : -Al Foreqls area to Aleppo PRMS. -FMS , Intermediate station and BVS Stations. |
| --- | --- |

    - Soil Testing Scope

| FMS stations Block valve sites Intermediate station Rivers Crossing | - Eight (8) test holes each (10m). - Two (2) test holes each (10m). - Two (2) test holes each (10m). - Two (2) test holes (20 m) One on each side. |
| --- | --- |
| Other crossing as required Along pipeline rout | - Two (2) test holes (10m) one on each side. - (1) test hole (10) m every 1 km In case if discrepancy arise, one test hole should take in the middle |

    - Methodology and Schedule

The CONTRACTOR should outline in his proposal the proposed methodology for the field investigation including type of equipment, number of personal and any anticipated difficulties.

A bar chart of similar schedule should be provided showing events, activities and milestones. Requirement for client input should be identified. Schedule to be based on completion of survey work, and delivery of all drawings prier to start work.

      - Project team

The proposal should identify key members of the project team, complete with a summary of each member’s qualifications and experience. Detailed resumes should be provided in an appendix. The key personnel should be identified according to the following categories:

Project Manager:	The person with supervisory and ultimate responsibility for the work.

Project Engineer:	The person responsible for executing the work, including arranging and supervising the field work.

Drilling Inspector:	The person who will log the test holes, taking the sample and supervising the drilling CONTRACTOR in the field.

Drilling CONTRACTOR:	The CONTRACTOR who will drill the test holes.

      - Office location

The location of the office from which the project will be handled should

be identified.

      - Description of the firm

The proposal should include a description of the firm, including curriculum vita

(C.V)

A description of recent projects, similar to this proposed here should be provided. References for these projects should also be provided.

    - Maine P/L Survey – AL FOREQLS to Aleppo PRMS.

COMPANY preliminary route for pipeline should be studied and approved from CONTRACTOR after that all necessary survey for pipeline (R.O.W) from:

AL FOREQLS to Aleppo PRMS.

Should be done by CONTRACTOR according to RFQ and specification.

Soil testing

      - Scope

This specification covers the minimum requirements for soil surveying for both pipeline construction and construction of facilities at sites associated with the pipeline.

      - Soil testing – general

1-	Drilled test holes shall be to a minimum depth of 10m where called for, 20 m test holes are required.

- The test holes shall be logged in the field and representative samples shall be retained for laboratory testing. Wherever possible, disturbed samples shall be retained at 0.75-meter intervals in each hole. Standard penetration tests are to be carried out at 1.5-meter intervals in each hole. Pocket penetrometer tests are to be carried out on disturbed samples of cohesive soils.

3-	A stand pipes piezometer shall be installed in at least one of the test holes at each of the BVS, SDV and FMS station sites. Water levels shall be noted at the completion of each hole and in the standpipe at the end of the day of drilling.

4-	The locations of the test holes should be determined in the field and the elevation of the ground surface adjacent to each test hole should be determined. A suitable benchmark will be established in the field by the soil Testing CONTRACTOR.

5-	Laboratory testing shall include the visual classification (using the Unified Soil Classification System) and moisture content determination of all samples (Angle of internal friction and cohesion: c) Atterberg Limits Test and grain size analyses shall be carried out on selected representative samples, as required, in order to confirm the visual classifications. Water soluble sulfate content tests should be carried out on selected representative samples, in order to establish the requirement for sulfate resistant concrete.

      - Reporting/Recommendations

The results of the investigation should be written in English and presented in a bound report. The report should outline the stratigraphy encountered at each of the sites and provide a summary description of major soil units, water levels and any other pertinent information. Design parameters and construction recommendations should be provided for the following:

- Driven steel piles if required

- shallow spread footings

- slab-on-grade floors

- soil properties for dynamic analysis

- tank foundations if required

- buried tanks if required

- suitability of native soils for containment dykes

- cement type for foundation concrete.

    - Survey and Right of Way

      - Scope

This specification covers the minimum requirement for the survey of the pipeline centerline, the Public Open Space, Permanent Right of Way and the Working Strip, and site survey for facility locations.

      - Pipeline centerline survey

1-	CONTRACTOR shall be responsible for all survey work including pre-construction and As-Built. CONTRACTOR shall survey the proposed pipeline Right of Way alignment using proven equipment and techniques. CONTRACTOR’s personnel shall have previous experience with the survey method and equipment to be used. COMPANY shall approve pre-construction route maps and alignment sheets.

2-	Field survey

CONTRACTOR shall perform a field survey of the pipeline centerline in order to:

- Provide sloped-distance of alignment for stationing and to provide the actual pipeline length.

- Provide horizontal distance of alignment, for Right of way acquisition

- Define the location of the proposed alignment for right of way acquisition and transfer the information onto aerial map and alignment sheets.

- Stake the line for visually locating of proposed alignment. Stakes shall be of such regularity that continuity will be maintained.

- Document topography and other physical features that may affect construction

- Provide a profile of the line for hydraulic design.

3-	Preservation of survey stakes

CONTRACTOR shall offset the centerline stakes as specified herein for clearing, grading and ditching. Other shall be held responsible for the preservation of all stakes and marks. Others shall bear the cost of replacing any stakes destroyed or lost and re-survey work required.

4-	Establishing survey markers

All property lines, restrictions, grades, lines, levels, and benchmarks shall be established and maintained by CONTRACTOR who shall be responsible for same.

5-	Profile

The CONTRACTOR shall profile of the survey centerline Elevations shall be obtained at all sharp breaks in slope, at high and low points, and at least every 100 meter on gently sloping terrain. Sufficient data on top and sub soil shall be obtained on active farm land, marsh land, road, railroad, wash, Wadi and river crossing profiles for permits applications and design criteria.

6-	Strip topography, crossing and underground structures

All significant topography and physical structures, which might affect pipeline construction and/or maintenance shall be located and included on the alignment sheets. There features include, but are not limited to, streams, washes, roads, trails, Wadi (s), buried cables, fences, cultivated areas, swamps or wet areas, rock outcrops, railroads, archaeological sites, vegetation and soil characteristics, quarries, or any other natural or cultural features deemed necessary by COMPANY.

For purpose of this survey, all topography of significance within the right of way shall be located.

CONTRACTOR shall make every attempt to identify underground pipelines, cables, etc. during the centerline survey. As direct by COMPANY’s representative, CONTRACTOR shall sweep the centerline route with pipeline detector. When an underground structure is encountered, it shall be identified with a description, size, depth and owner if at all possible.

True (Geodetic) bearings shall be used for alignment, permit and property plot drawings.

      - Mapping and drawings

1-	Alignment sheets

CONTRACTOR shall draft the survey alignment and also include land ownership with ties to corners and topographic features. All significant topography noted above shall be included on the alignment sheets. CONTRACTOR shall plot the centerline of the alignment on the aerial map immediately following the survey.

Alignment sheets shall be consistent with the example attached.

2-	Property plats

CONTRACTOR shall prepare a plat drawing for each property location and for every property parcel crossed by the proposed route. Plats shall locate surveyed alignment with respect to title information. Legal description shall be furnished with each plat. Each plat shall be certified correct by the CONTRACTOR. Scale of plat shall be approved by COMPANY.

3-	Permit drawings

CONTRACTOR shall prepare permit drawings for all road and railroad crossing, for river crossing, and crossing of active farm land for the propose of acquiring permits and for crossing government lands. All permit drawings shall be in accordance with the specification of the requesting agency.

- Preference will be given to CONTRACTOR who can supply alignment sheets and other drawings prepared using AutoCAD, Release14.

      - Public open space, permanent right of way and working strip

1-	The permanent right of way for the pipeline will be a strip of land 20 meters wide in total, 8 meters for back filing material and 12 meters for working route. An additional 15 meters on both sides of the Right of Way is designated as Public Open Space to safeguard the pipeline after construction.

2-	CONTRACTOR may make use of a Working Strip, having a width of 20 meters (including a continuous 6 meters wide vehicle access strip). Variations, if any, in this width caused by local conditions shall be indicated on the drawings or instructed to CONTRACTOR by COMPANY.

3-	CONTRACTOR shall, before starting any surveying operations, familiarize himself with any requirements by Authorities for work along the Pipeline Route or in connection with the use of or access to other lands or roads for surveying purposes.

4-	CONTRACTOR shall notify COMPANY seven (7) days in advanced of survey about road, pipeline, cable, stream and Wadi crossing and crossings of other existing structures, if any.

5-	CONTRACTOR shall not commence survey work on such crossings before having obtained approval from the Authorities and landowners concerned to the satisfaction of COMPANY. The crossing shall be surveyed to meet all the requirements and conditions of the permits, issued by the Authorities concerned.

6-	In the absence of any specific requirements by Authorities, CONTRACTOR shall comply with COMPANY instructions.

7-	Where the permanent right of way or Public Open Space comes within 25 meters of an existing line, CONTRACTOR shall identify the line on drawings.

8-	Equipment may be operated up to 2 m from existing facilities provided, thesis facilities are properly located and indicated by CONTRACTOR.

      - BVS  survey

CONTRACTOR shall be responsible for a survey of each BVS in order to:

1-	Prepare a scaled (1:300) plot drawing on AutoCAD showing all surface and features (not less than 900 m2 square).

2-	Identify any feature (manmade or natural) within 100 m of the BVS which may influence design i.e. water course, Wadi, significant change in grade or structure.

3-	Site cleans up.

      - FMS-COMP

CONTRACTOR shall be responsible for a survey of FMS-Compressor and SDV area in order to:

1-	Establish a suitable benchmark and determine geodetic elevation of same.

2-	Establish grade elevation on a 10 m grid over the site.

3-	prepare a scaled plot drawing in accordance with 8.3.5.5.1

4-	Identify any features in accordance with 8.3.5.5.2

- Survey locations of proposal test hole locations for soil survey.

6-	Site cleans up.