SCHEDULE SPS-12

PROJECT SPECIFICATION FOR FIBER OPTIC CABLE LAYING

TABLE OF CONTENTS

1.0	SCOPE.

2.0	GENERAL

3.0	CABLE LAYING

4.0	CROSSING.

5.0	BACK FILLING.

6.0	MARKERS

7.0	ROUTES AT PLANT AREAS.

8.0	CABLE DRUM HANDLING.

1.0	SCOPE.

This specification is intended to define the minimum requirements for the laying of a fiber optic cable for the ARAB GAS PIPE LINE - PHASE II.

2.0	GENERAL

2.1	The proposed fiber optic cable has a steel tape armoring and shall be laid in the same trench with the ARAB GAS PIPE LINE PHASE II from Gas coordination center area to the FMS metering station ..

The only deviation allowed from laying the cable in the same trench as the pipeline is within the confines of the metering or BVS plots.

2.2	Cable laying installation details are shown on attached sheets. Since the cable shall be laid in the same trench with the pipeline, the lying of the cable shall match with the lying of pipeline.

2.3	The Contractor shall ensure that during the subsequent pulling of the cable through the conduit that adequate precautions are taken to protect the cable against scuffing and over tensioning.

2.4	Contractor shall prepare sheets for all type of crossings, complete with topographical details.

3.0	CABLE LYING:

3.1	The trench layouts are indicated in the attachments. The excavation will be carried out either by hand or by engine. Particular care shall be taken when approaching over ground and underground services in order to avoid damaging them.

The trench shall be enlarged at the splicing points to carry out the splicing work and to perform the cable positioning according to the attachments.

The trench shall be drained during the splicing work.

The enlargement shall be carried out at the same side in respect to the cable for the whole route selected in order that joints and loading boxes result at the opposite side in respect to the permanent markers.

3.4	All works concerning the preparation of the lying bed shall be well coordinated with lying of the pipeline in order to avoid any damage to the pipeline coating.

Before the cable lying operation, a 10-cm depth laying bed shall be prepared.

It shall be made by sand or riddled earth by a sieve having meshes not exceeding 15 mm2 or by natural soft ground not requiring riddling.

3.3	The layout of the cable sections should be done in such a manner that at each splice point, a merger of a clockwise rotation head and an anti-clockwise rotation head of the fibers occurs.

If the cable lying is carried out by two spreads proceeding in opposite directions, the meeting point between the two sections shall coincide, if possible, with a splice point.

The cable can be laid using mobile reel system or fix reel system in relation to the characteristics of each section where the cable shall be laid (excavation partially open or crossing etc.).

In case of mobile reel system the cable shall be laid directly to the bottom of the ditch on the bed made in advance.

The cable shall not be over-stressed during the unwinding procedure by doing the unwinding by hand.

The support of the reel shall be connected in a locked position with the carrier or means of transportation in order to avoid that any rotation on the vertical plane passes through the reel’s axis.

In case of a fixed reel system the traction stress shall not be more than the allowable resistance of the cable, established by the cable manufacturer.

The pin of the reel shall be put on the proper rack.

The cable shall be laid near the trench and later on lowered down on the bed by hand. This is in order to avoid damage of the bed by the operators.

In both cases, the axis of the laid cable shall appear as a sinusoidal line in respect to the horizontal plane in order to avoid, after back filling, damages due to ground settling.

At the points where the pipeline changes its direction such that during the laying the pipe would come near the trench wall, a widening of the trench shall be provided thus assuring the minimum distance necessary between the cable and the pipe.

Every time the cable lying will be interrupted the cable shall be protected by suitable means.

If the splicing works do not follow immediately after the laying, all cable ends shall be buried to avoid any damage or theft and the point properly marked in order to enable the splicing team to find the exact spot.

4.0	CROSSING.

If crossing occur, they shall be carried out in accordance with the attachments.

4.1	General:

Some types of crossing the cable shall be protected by steel casing having a diameter not less than 8 cm.

Casing pipes shall be made of galvanized steel duly painted.

For longer crossings the protective pipes shall be jointed using proper threaded couplings assuring the waterproofing.

If the cable laying does not follow immediately the protective pipe installation, a pulling rope shall be provided to facilitate the subsequent cable pulling.

After its installation, the casing pipe shall be cleaned on the inside from any kind of material (earth, stones, etc.).

The ends of the casing pipes shall be closed to prevent foreign objects inside the pipes. For the same reasons the casing ends shall be closed after the introduction for the cable, using a proper plug.

Normally, the cable shall not be cut for crossings; Cuts will be allowed for long crossings and in case the cross is at the middle of the cable length.

4.2	Pipeline Crossing:

Normally, the crossing of the pipeline shall be avoided. In particular points the cable shall cross the pipeline to joint the section of the cable laid or to be laid at the opposite side of the pipeline.

The Contractor shall be surrounded by 10 cm of sand or riddled earth as described in par 3.2

4.3.	Road, Railway, River Crossings:

In swamps, flooded areas, etc. if concrete weighting is applied to the pipeline, following events will occur:

a)	Continuous weighting applied to the pipeline before it’s laying APPENDIX I shall govern.

b)	Lean concrete weighting: steel casing shall be incorporated in the concrete cast. Normally, steel-casing length shall be less than 100 m.

Where maximum allowed length be insufficient, interruptions among sections shall be provided to allow section by section cable pulling.

c)	Block weighting: same procedures for normal laying apply, the cable maintaining the normal position in respect of the pipeline.

Steel casing necessary for the crossings shall be provided and installed, cleaned and closed as needed, ready for cable pulling.

4.4	Crossing of Other Underground/Over ground Services:

Where crossing of existing underground pipelines and cables, pipes on sleepers, overhead telephone and electric lines, etc. occur, the same requirements for normal laying apply, the cable maintaining the normal position in respect of the pipeline.

4.5.	Bridge Crossing:

At overhead crossing on existing bridge the cable shall be protected for the whole over ground route by steel casing pipe clamped to the bridge structure.

The Contractor shall design and carry out the crossing at its full care and responsibility including supply and installation of the steel casing.

5.0	BACK FILLING.

5.1 		Back filling procedure:

After the cable is laid, a not less than 10 cm. Depth cover shall be provided.

It shall be made by sand, riddled earth or by natural ground not requiring riddling as stated on par 3.2

On the cable no mechanical protection shall be normally provided with the exception of splicing points and stated particular cases.

Warning tape shall be provided above the cable.

The Joints, relevant cable rings and bricks shall protect other underground works.

Bricks, standard type, shall be laid flat and approached by their own smaller side.

If some difficulties appear for brick provision, concrete slab or equivalent material could be utilized.

At the spicing points, the contractor shall also take care to backfill the excavation and to restore the relevant area with special regard to road borders and full repair of damaged works.

5.2	Cable in Separate Trench:

This applies only within BVS – FMS- launcher – receiver- intermediate station and the Syrian Jordan border. All back filling works shall be at full Contractor’s charge and responsibility.

The cable shall be covered by sand and mechanically protected by bricks for its whole length in the same way stated in par 5.1

No bricks shall be back filled with earth without stones or pebbles.

The protection of the joints and other underground works shall be realized as stated in par 5.1

After lying, the Contractor shall take care of cleaning the working track and removing all debris.

The land restoration shall be also at the Contractor’s care with special regards to road borders and full repair of the damaged works.

6.0	MARKERS

All joints, loading boxes and other underground works shall be marked by proper plates which shall be installed on the markers.

The markers shall indicate, in different form and with progressive number:

-	The planned joints.

-	The joints performed both for incidents or required by particular condition.

-	The planned loading boxes.

-	The possible cable rolls as spare or line simulation.

-	The dependent repeaters.

Such information shall be duly reported on installation “as-built” drawings.

Cable In the Same Trench with a Pipeline:

Cable markers shall be positioned along the route in line with the pipeline markers.

7.0	ROUTES AT PLANT AREAS.

The cable route inside plant areas from the entrance point at the area fence up to the building where the cable must be terminated shall be agreed with the plant owner.

All requirements stated in the previous paragraphs. For cables in separate trench shall be followed.

Markers shall be provided along these cable routes.

8.0	CABLE DRUM HANDLING.

8.1	General:

All cables will be coiled on wooden drums, lagged with stout, closely fitting battens to effectively prevent damage to the cable during storage and transportation.

At least manufacturer’s name-contract number - drum number - description of cable in short, stating the type, number of fibers - length in meters - weight and direction of rolling will be plainly marked on the outside of each drum.

Although cable drums give a strong impression they contain a highly valuable, but also a rather vulnerable cargo which cannot take rough handling.

Therefore, the handling of cable drums requires utmost care.

In carrying out the work, personnel must comply with the safety regulations.

This Installation instructions cover in particular considerations in handling of cable drums.

8.2	Standing/Rolling:

-	Always keep the drums standing upright.

-	Always roll in the direction of the arrow marked on the flanges. Rolling of cable from is only allowed over short distances. For long distances a cable drum carrier must be used.

8.3	Lifting by Crane or fork-lift:

-	Cable drums may be lifted by crane or fork-lift

-	When using a crane:

Always hoist the drums by pulling the sling or a steel bar through the axle-holes in the flanges.

-	When using a Fork-lift:

Always put the two forks under two flanges.

8.4	Loading/Unloading/From Truck or Wagon:

-	Preferable load by crane or gin or by means of a winch mounted on the truck.

If a winch mounted on the truck has to be used, a steel cable should be laid around the battens of the drums.

-	The drum can be rolled down slowly on a pair of beams from the truck to the ground.

Never drop cable drums from the truck either directly onto the ground or onto a buffer.

8.5	Stowage/Securing:

-	To secure stowage of the drums, use two wedges in the hells of the flanges.

Never drive nails into the drums.

8.6	Storage:

-	Always place cable drums on a flat and firm surface. Use a Fork-lift or crane to place drums into position.

Cable drums must be lifted into position, not bumped.

-	Avoid extreme heat and exposure to direct sun light.

-	Place cable drums in rows, with marked flanges to the same side and at intervals allowing proper transport.

The cable drums must be inspected before storage and in case of cable damage found owner staff informed .

8.7	Piling:

-	Battens around the drums are not suited to withstand heavy burden.

Avoid denting of the battens.

Only drums with same width may be piled flange on flange.

Drums may only by piled standing upright.

-	In case of piling lower layer of drums must be secured in front and rear over full width of drums by beams nailed to the floor.

APPENDIX I:

PEEL STRENGTH OF ALUMINIUM FOIL ON CABLE SHEATH:

Introduction:

This test determines the force required to separate the polymer and the aluminum at a specified speed using a suitable tensile testing machine.

The peel strength of each test piece is defined as the mean of at least five readings taken at regular intervals along the test piece. The peel strength of the material under test is the mean of the results obtained on five test pieces.

Test Pieces:

Parallel-sided test pieced 150 mm long and 14 + 2 mm wide shall be cut longitudinally form the cable sheath in one operation using a sharp punch, operated from the aluminum side of the sheath. Test pieces shall exclude the overlap.

Testing Machine:

An electrically driven constant rate of extension tensile testing machine shall be used. The rate of separation of grips shall be between 100 and 125 mm. Per minute.

Procedure:

Peel approximately 50 mm of aluminum form one end of the test piece. Insert the aluminum in the one grip of the tensile testing machine and the polymer in the outer grip and record the force required to separate polymer and aluminum at regular time intervals of 8 sec along the length of the pieces shall be tested.

Calculation of Results:

For each test piece tested above, ignore the first and last readings and calculate the mean of the remainder and express the results in terms of N per 25 mm of width.

Calculation the peel strength of the material by determining the mean of the five results thus obtained.

APPENDIX II:

Optical Fiber Cable - Various Tests :

Type tests, as relevant standard shall be carried out to prove conformance with the specifications generally.

Type Test 	:

Fiber

-	Testing of mechanical properties.

-	Temperature Cycling (fiber supplier).

- Tensile proof - testing (fiber supplier).

Cable

-	Temperature Cycling generally according to method IEC-794-1-F1.

-	Crush Resistance test according to method IEC-794-1-E3.

-	Tension test generally according to method IEC-794-1-E1.

-	Material tests.

Physical Properties:

Tests on incoming goods, to verify the integrity of the material manufactured.

Cable dimensions:

Tube dimensions 	:	1.0 inner diameter.

1.5 outer diameter.

Tolerance 	+ 10%.

Sheath thickness	:	nominal 2.0 mm.

Average 1.8 mm.

Outer diameter	:	Nominal 12.5 mm.

Properties of dimensions shall also be tested random during the manufacturing process.

Acceptance test (Final Inspection):

-	Attenuation coefficient measurements at each fiber < 0.22 db/km.

-	Dispersion (guaranteed value of fiber supplier ).

-	General inspection on construction conformance.