ANNEX - 1

CLASSIFICATION OF EQUIPMENT AND SPARE PARTS

FOR NORMAL OPERATION

  - CLASSIFICATION OF PLANT EQUIPMENT

When considering the ordering and stocking of spare parts for normal operation of the plant, the importance of the parent equipment in efficient plant operation and its future use should be taken into account. The classification of each piece of equipment has to be established by the Operating Division of Company. Equipment should be classified as follows:

1.1	Vital Equipment

Equipment of which a breakdown would mean an immediate and serious interruption of vital operation and with which no risk in the ordering and stocking of spares can be justified. For this class of equipment, the stocking of insurance items (I) shall be considered.

1.2	Essential Equipment

Equipment used in primary operations, but with which a calculated risk can be taken in the ordering and stocking of spare parts, in view of the fact that either stand-by capacity is built into the plant or spare units of equipment are installed. For the class of equipment normally only consumable parts © are stocked and possibly, depending on circumstances, some of the less expensive replacement parts.

  - Auxiliary and General-Purpose Equipment

Equipment which owing to its function can be allowed to remain temporarily out of operation without having a serious effect on operations. Under this heading also comes equipment of which there are a large number of identical units in use, thus ensuring a sufficient degree of protection in the case of failure of a unit. For this class or equipment normally only consumable parts © are stocked.

  - CLASSIFICATION OF AUTOMOTIVE EQUIPMENT

It is not always practicable to treat automotive equipment in the same way as plant equipment. Consequently, the following classes for automotive equipment may be more practical:

2.1	Light Fleet Units

Mass produced units with a comparatively short life which it is found more economical to replace after a specified mileage rather than to overhaul.

For this type of unit it may be desirable to stock consumable parts © and certain assemblies to facilitate quick repair in the event of premature breakdowns.

2.2	Heavy Fleet Units

Usually non-mass produced units with high initial costs and longer life. Depending on the size of the fleet and type of service they provide, some of these vehicles may be regarded as “vital equipment” and consequently stocking of insurance item (I) shall be considered.

2.3	Trailers

Usually have a long life. Therefore a few consumable parts (C) and replacement parts (R) shall be stocked.

3.0	CLASSIFICATION OF SPARE PARTS

Spare parts are divided into three classifications:

- Consumable parts (C)

- Replacement parts (R)

- Insurance items (I)

Definitions of the three classifications are given below and when interpreted correctly they should help to avoid over-ordering of spare parts and of replacement parts in particular.

The classification of spare parts (C, R or I) as agreed upon by operating and purchasing departments, should be indicated in the appropriate records, and any subsequent reclassification should be mutually agreed.

3.1	Consumable Parts (C)

Spare parts (including general accessories) of which it is known (or, for new equipment, assumed) that they will be required regularly because of wear or deterioration.

Purchasing department will be responsible for the stock replenishment of consumable parts (C) based on estimated future consumption, taking into account estimated future consumption and planned future operations as advised by the operating departments concerned. This activity will be automatically actioned by computer link from the field.

3.2	Replacement Parts (R)

Spare parts or complete assemblies which are individually ordered on the basis of prior inspection or against planned overhaul schedules (where sufficient operating experience has been gained on which to base reasonably accurate forecasts of requirements).

Replacement parts (R)required as a result of prior inspection are requisitioned only by the operating department. Where overhaul schedules are established, and details of requirements per overhaul are provided by the operating department, purchasing department is responsible for ordering the necessary replacement parts in sufficient time to meet the overhaul schedule.

It will be appreciated that the classification of an individual spare parts item does not necessarily depend on the part itself, but rather on the regularity and frequency of its consumption. Thus parts, which normally would be classified as replacement parts (R) could, because of prevailing operating conditions and resulting consumption pattern, be classified as consumable parts (C).

  - Insurance Items (I)

Spare parts or complete assemblies (usually of high value, such as a turbine rotor, compressor rotor), which are not normally required for ordinary maintenance because of wear or deterioration and thus for which at time of purchase, there is no foreseen consumption. However, they may be required in the event of an unforeseen breakdown, to avoid a lengthy shutdown of a vital piece of equipment.

Prior to ordering action, a careful investigation should be made into alternative methods of minimizing the financial consequences to the Company, should such an unforeseen breakdown occur. The possible alternatives to be studied, and individually evaluated, are:

- repairing the damaged part-either on site or at a local factory. This will involve operational losses whilst the plant is out of commission plus cost of repairs plus cost of freight (if applicable).

- Ordering a new part and awaiting its arrival before repairs of the plant can be carried out. This will involve larger operation losses (since factory delivery of new parts is generally long) plus cost of part, plus cost of air or sea freight.

- Ordering a new part for stock in advance of a breakdown so that it is available for immediate installation when required. This will involve the landed cost of the part plus loss of interest on investment for the period it is in stock without use (with the possibility of complete write-off it the part is in fact never used).

The purchase of an insurance item (I) for stock is justified only if alternative 3 above produces the lowest total cost. In this connection the financial implications of capitalizing insurance items should be taken into account.

Operation would be responsible for initiating any orders for insurance items.

If it is decided to order an insurance item, it is recommended that this be done simultaneously with the main equipment, since this, as a rule, will be more economical and will enable complete shop testing. A rotor assembly shall be ordered complete with its bearings, coupling, seals, etc., as a set. The complete set shall be installed and not a part thereof.

Note:	Where local regulations allow, the high value insurance items should not be included in the spare parts stock value, but capitalized with the related asset (parent equipment).

It is recommended that the capitalized insurance items remain physically and administratively under the control of materials department although no values will be included in the materials stock account.