Packing Details

- Heavy and large pieces of equipment dimensionally and traditionally considered too large for crating, such as, but not limited to columns, towers, reactors, condensers, exchangers, compressors, turbines, etc.:

- All equipment shall be thoroughly cleaned of foreign matter and drained of water before shipment.

- Shall have all exposed machined surfaces cleaned and covered with a layer of anti-corrosive material which can withstand exposure to moisture and hot temperatures (45C) for a period of 1 ½ years

- All orifices shall be made moisture and dust proof by sealing off with wood, metal, plastic plugs or in some cases weatherproof tape.

- Flange openings shall be coated with a layer of anti-corrosive material then closed off with wooden, metallic, plastic or composition board fastened in place with tape or non-rusting devices.

- If deemed necessary to maintain the manufacturer’s warranty, the addition of a drying agent or possibly a controlled interior atmosphere, sealed and supported with bottled gases shall be used.

- An interfacing of wood and neoprene shall be placed between a piece of equipment and a metal shipping cradle or frame to protect the surface from scratching or gouging (2 cm wood/1 cm neoprene thickness).

- 1cm thickness of neoprene must be placed between the equipment surface and encircling hold-down metal bands.

- Lifting lugs or slinging points must be provided and well identified for handling in transit.

- One (1) copy of the packing list and technical data shall be placed in a sealed waterproof envelope and firmly fastened to the non-working surface of the equipment in such a fashion that it will not be torn away.

- Mechanical, electrical, electro-mechanical equipment such as, but not limited to electric motors, pumps, control valves, gear motors, control/measuring equipment, etc.:

- Shall be “shrink wrapped” with a drying agent enclosed inside the package. (Protruding parts shall be protected to avoid perforating the tight plastic wrap.).

- The item shall then be rigidly fastened inside a wooden case, totally immobilized by being bolted down to the base, using carriage bolts and kept in a stationary position by blocking, bracing or shimming. (Lag bolts not to be used for tie-down or anchoring.) (If shrink wrapping’ not possible, the crate must be lined inside with waterproof material and a drying agent enclosed).

- Any case weighing in excess of 40 kg must be so constructed as to have bottom wooden skids of a thickness no less than 7 cm to permit handling by crane.

- The case may be nailed or bound with steel strapping and must be made of good quality wood, to withstand stacking of at least three (3) times its own weight.

- One (1) copy of the packing list and technical data shall be enclosed in a sealed waterproof envelope and fastened to one (1) of the inner sides of the case.

- One (1) copy of the packing list shall be fastened in a sealed waterproof envelope to the outside of the case and covered by a tin plate, on which is printed “Packing List”.

- Packing or items having different destination codes or Purchase Order numbers in the same case is not permitted.

- Electrical, electronic, computer, control and regulation equipment of high fragility and sensitive to shocks and vibrations:

- Shall be “shrink wrapped” with drying agent enclosed; wrapped in crepe cellulose wadding or equally efficient cushioning material then placed in a wooden case. The case will now be an inner case to a larger wooden case having bottom skids as previously described in b. iv.

- Between the inner and outer wooden cases there shall be a shock absorbing system of springs, pneumatic cushions or mattresses calculated to withstand the weight, volume and fragility of the interior goods.

- One (1)-packing list will be inside and another outside the crate, as described in b-v and b-vi above.

- Packing or items having different destination codes or Purchase Order numbers in the same case is not permitted.

- Straight Piping:

- Internal protection of the pipe will be attained by spraying with oil or anti-corrosive product.

- Pipe ends will be sealed with plastic, wooden, metallic plugs or waterproof material well secured in place.

- The pipe exterior shall be coated with an anti-corrosive varnish or lacquer through which stenciled information will be visible and legible.

- The pipe will be bundled in quantities not to exceed a weight of 4.5 tones, length of 13 m, width/height 1.2 m.

- The bundles will be bound with metal strapping encircling the pipe,  with wood stripping between the pipe and banding.

- Thin strips or wood (dunnage) will be placed between the layers of pipe.

- The tension of the metal banding shall be sufficient to exert enough pressure to cause friction between metal and wooden surfaces great enough to prevent longitudinal movement.

- Spacing between the encircling bands shall not be greater than 1.5 m, nor will they be more than 0.3 m from the pipe ends.

- One (1) copy of the packing list complete with a mill certificate shall be placed in a sealed waterproof envelope and attached to the bundle in a protected area away from slinging locations.

- Pipe Fittings, Flanges and Valves:

- Pipe fittings will be placed in waterproof lined wooden cases in such a manner than movement within the case will be minimized to the greatest possible degree (as in b. I and ii above). Weight of each case not to exceed 4.5 tones.

- Flanges and valve flanges will have surfaces coated with an anti-corrosive varnish, then they will be packed in waterproof lined wooden cases (as in b-i and ii above) in such a manner that the flange faces will be separated from other metal by wood or fiber board. Weight of each case not to exceed 4.5 tones.

- Each case will contain only one size, one grade and one kind of fitting, flange or valve.

- Packing or items having different destination codes or Purchase Order numbers in the same case is not permitted.

- One (1) copy of the packing list complete with a mill certificate shall be placed in a sealed waterproof envelope and attached to the inner side of the case lid.

- One (1) copy of the packing list shall be fastened in a sealed waterproof  envelope to the outside of the case and covered by a tin plate, on which is printed “Packing List”.

- Notification of Packaging:

When any cargo exceeds 25 tones in weight, 19 m in length, 3 m in height or 2.5 m in width, VENDOR shall fax to SPC a sketch of the oversize cargo sixty (60) days before delivering each shipment. If any cargo contains flammable, hazardous or dangerous items, VENDOR shall also fax an English description of the items giving its name, characteristics, special handling and protective measures and procedures in case of accidents. For any cargo which requires special handling due to storage temperature considerations, VENDOR shall also fax an English description stating special precautions to be taken during transportation.

This information shall be taken as the basis for arranging transportation and storage of the cargo and it is the responsibility of the VENDOR to advise SPC so as to allow SPC sufficient time to make all necessary and required arrangements.

- Containerization:

The use of containers for ocean shipment is encouraged where possible, however the cubic space of the container must be fully utilized.

- If containers are to be used, calculations should be made in advance to determine the size of cases planned to go in the container, to fully use the space.

- One (1) copy of all packing slips must be placed in a sealed waterproof envelope on the outside of the container.